# Task Assignment

Thank you for applying for the Junior Scientist - Edge-AI Accelerators at Silicon Austria Labs. We appreciate your interest in joining our team and would like to invite you to complete the next step in our selection process.

To help us better understand your skills and approach, we have prepared a task assignment related to the role you're applying for. This task is designed to assess your problem-solving abilities, creativity, and the specific skills relevant to the position. Please find the details of the task below.

We encourage you to take your time with the assignment and showcase your best work. Feel free to ask any questions or request clarifications if needed.

Task Deadline: [6th Feb, 2026]

Submission Instructions:

Please submit the HDL code and testbench, written preferably in SystemVerilog (but VHDL and Verilog is fine as well).  If done using any HLS tools, please explain your method/steps in the supporting document/presentation. The supporting document/presentation may include the block diagrams and simulation results that you consider necessary to understand your submission.

We look forward to reviewing your submission!

T1: Hardware Design – The MAC Unit (Compute Core)

In AI accelerators, the Multiply-Accumulate (MAC) operation is the fundamental building block.

- Task: Design a Parametric Dot-Product Engine in SystemVerilog/Verilog/VHDL.

  - Result=i=03(Ai x Bi)

- Specifications:

  - Input: Two vectors of length N=4.

  - Data Type: 8-bit signed integers (INT8) for both weights and activations for quantized Edge-AI.

  - Output: A single accumulated result. Ensure the bit-width of the accumulator prevents overflow.

  - Architecture: The design should be pipelined to achieve high clock frequency.

  - Deliverables: RTL code, a simple testbench showing a valid dot-product calculation, and a brief explanation of the pipeline stages chosen.

T2: System Integration – Memory Mapped Interface

Define the interface and the Control State Machine.

- Task: Design a simple Control/Status Register (CSR) Interface for the Dot-Product Engine from T1.

- Specifications:

Define the Register Map and Controller for the unit in T1.

- Register Map Table: Create a table showing the memory addresses a CPU would use to write the 8 input values and read the 1 final result.

- Finite State Machine (FSM): Provide a diagram or a brief description of a simple FSM that manages the process.

- Requirement: Provide a block diagram showing how the data flows from a control unit, through the interface, into the MAC unit.

T3: Processor microarchitectures

The figure below presents two different processor microarchitectures. Please concisely explain the clear differences between the two, elaborating on the second one. We shall revisit these architectures during the interview.